COAL

Assignment – 03

By

Sharjeel

L1F24BSCS0506

CODE

.model small

.stack 0x100

.data

msg db 'Hey, I am Sharjeel Ahmad currently pursuing BsCs at University of Central Punjab, Lahore. This is assignment - 03 of COAL...!$'

row dw 24

col dw 0

.code

main proc

mov ax, @data

mov ds, ax

mov ax, offset msg

push ax

mov ax, row

push ax

mov ax, col

push ax

call printString

add sp, 6

mov ax, 0x4c00

int 0x21

main endp

;function definition

printString proc

push bp

mov bp, sp

push ax

push bx

push cx

push dx

push si

push di

mov si, [bp + 4] ;offset string

mov ax, [bp + 6] ;row

mov bl, 80

mul bl ;ax = row * 80

add ax, [bp + 8] ;ax = row * 80 + col

shl ax, 1 ;ax = (row * 80 + col) * 2

mov di, ax ; di = video memory offset

mov ax, 0xB800 ;video memory segment

mov es, ax

printLoop:

mov al, [si]

cmp al, '$'

je exit

mov es:[di + 0], al

mov byte ptr es:[di + 1], 0xb3      ;grey character with black background

add si, 01

add di, 02

cmp di, 4000

jb printLoop

call scrollUpward

mov di, 3840

jmp printLoop

exit:

pop di

pop si

pop dx

pop cx

pop bx

pop ax

pop bp

ret

printString endp

;function definition

scrollUpward proc

push si

push di

push cx

push ax

mov ax, 0xB800

mov es, ax

mov si, 160             ;2 * 80

mov di, 0

mov cx, 1920            ;80 * 24

scrollLoop:

mov ax, es:[si]

mov es:[di], ax

add si, 2

add di, 2

loop scrollLoop

mov di, 3840

mov cx, 80

clearWithSpace:

mov es:[di], ' '

add di, 2

loop clearWithSpace

exitScroll:

pop ax

pop cx

pop di

pop si

ret

scrollUpward endp

______________________________________________________________________________

Screen Shots

At r = 0, c = 0

At r = 24, c = 0