University of Central Punjab -FOIT

COAL Fall 2025 
Quiz 1

Q1. Calculate the logical and physical address.

Segment: A3F2		offset: 00B5

Logical address: A3F2:00B5

Physical Address: A3FD5

Q2. Give the proper reasoning for the validity and invalidity of the following instructions.

- MOV 0005h, AX

Immediate value can’t be a destination.

- MOV IP, BX

IP register can’t be changed directly.

- MOV ES, 100h

Cannot move an immediate value directly into a segment register (like ES).

- MOV AX, ABCDH

Valid

- MOV [BX], [SI]

Both operands can’t be memory at the same time.

Q3. Write a code snippet in assembly language to add the values 10h and A2h and place the result in register DX.

model small

.stack 100h

.data

.code

MOV AL, 10H

ADD AL, A2H

MOV DX, AX

.exit