University of Central Punjab -FOIT

Fall 2025 – C13

Quiz 4				Total Marks = 10				CLO = 2

- You have an integer counter initialized to 128. Using inline assembly, divide counter by 4 using a shift-right instruction and store the result back in counter. (5)

#include <iostream>

using namespace std;

int main() {

int counter = 128;

__asm {

mov eax, counter  ; move counter into eax

shr eax, 2        ; shift right by 2 bits (divide by 4)

mov counter, eax  ; store result back into counter

}    cout << "Result: " << counter << endl; // should print 32

return 0; }

- Using inline assembly, add 7 to every element of the 2×2 integer array matrix and display the results. (5)

#include <iostream>

using namespace std;

int main() {

int matrix[2][2] = { {1, 2}, {3, 4} };

__asm {

lea esi, matrix       // Load address of matrix into ESI

mov ecx, 4            // Total number of elements (2x2 = 4)

loop_start:

mov eax, [esi]        // Load current element

add eax, 7            // Add 7

mov [esi], eax        // Store back

add esi, 4            // Move to next element (sizeof(int) = 4)

loop loop_start       // Repeat ECX times

}    for (int i = 0; i < 2; i++) {

for (int j = 0; j < 2; j++) {

cout << matrix[i][j] << " ";

}        cout << endl;

}    return 0; }

Quiz 5				Total Marks = 10				CLO = 3

- Two MMX registers contain: (2)

mm0 = 0x9000 0xF000 0x4000 0x5000

mm1 = 0x9000 0x2000 0x6000 0x4000
What will be the result of paddw mm0, mm1?

0x2000 0x1000 0xA000 0x9000

- Two MMX registers contain: (2)

mm0 = 0x2000, 0x8000, 0x4000, 0x7000

mm1 = 0x1000, 0x2000, 0x8000, 0x9000

What will be the result of pmulhw mm0, mm1?

0x2000 0xF000 0xE000 0xCE00

- Two MMX registers contain: (2)

mm0 = 0xF0F0, 0xAAAA, 0x1234, 0xFFFF

mm1 = 0x0F0F, 0x5555, 0x00FF, 0x0F0F

What will be the result of por mm0, mm1?

0xFFFF 0xFFFF 0x12FF 0xFFFF

- Two MMX registers contain: (4)

mm0 = 0x0102030405060708

mm1 = 0x0807060504030201
What will be the result of the following sequence of instructions?

__asm{

paddw mm0, mm1

psrlb mm0, 2

}

Paddb: 0x0909 0x0909 0x0909 0x0909

Psrlb: 0x0202 0x0202 0x0202 0x0202